DESIGN JUSTIFICATION REPORT

ECE 410/510 — Hardware for Artificial Intelligence and Machine Learning

Portland State University | Spring 2026

Project: BF16 Conv2d Accelerator — 32x32 Weight-Stationary Systolic Array

PDK: SAED14nm RVT | Tool: Cadence Genus 17.14-s037_1 | Clock: 500 MHz

INDEX

| Section No. | Title | Page No. |
| --- | --- | --- |
| 1. | Problem and Motivation | 2-3 |
| 2. | Roofline Analysis | 4-5 |
| 3. | Precision and Data Format | 6 |
| 4. | Dataflow and Architecture | 7-8 |
| 5. | Hardware Interface | 9 |
| 6. | Verification | 10-11 |
| 7. | Synthesis Results | 12-13 |
| 8. | Benchmark Results | 14-15 |
| 9. | What did not work | 16-17 |

SECTION 1: PROBLEM AND MOTIVATION

The target application is a hybrid anemia-detection model (ResNet18 backbone + AttentionFusion MLP) trained on the AneRBC peripheral blood smear dataset (12,000 images: 6,000 healthy, 6,000 anemic). The model was profiled running on an Intel Core i5-10210U laptop CPU (Windows 11, PyTorch 2.11.0, CPU-only) over 10 full training epochs, each epoch processing 9,600 training images in 300 batches of 32.

Wall-clock timing (time.perf_counter, full pipeline including data loading, feature extraction, validation, GradCAM, and logging):

1.1 Wall-Clock Timing (time.perf_counter, full pipeline)

| Run | Time |
| --- | --- |
| Run 1 | 2762.6 s |
| Run 2 | 2670.4 s |
| Run 3 | 2595.1 s |
| Run 4 | 2537.6 s |
| Run 5 | 2552.6 s |
| Run 6 | 2497.8 s |
| Run 7 | 2508.4 s |
| Run 8 | 2481.1 s |
| Run 9 | 2860.9 s |
| Run 10 | 2719.2 s |

Median wall-clock: 2573.9 s (42.9 minutes) per epoch

Mean: 2618.6 s

Std deviation: 128.8 s

1.2 cProfile Kernel Identification

cProfile of the training loop (hybrid_train_one_epoch, 300 batches, 1266.6s total profiling runtime)

Identified torch.conv2d as the dominant kernel:

| Function | Cumtime (s) | % of Total |
| --- | --- | --- |
| run_backward (autograd engine) | 732.7 | 57.8% |
| HybridModel.forward | 421.6 | 33.3% |
| resnet.forward | 324.8 | 25.6% |
| torch.conv2d <-- DOMINANT | 292.9 | 23.1% |
| torch.batch_norm | 60.3 | 4.8% |
| torch.max_pool2d | 47.1 | 3.7% |

torch.conv2d (and the backpropagation gradients flowing through the same Conv2dweights) accounts for 23.1% of runtime directly and 57.8% including backward. Combined Conv2d forward + backward accounts for 74.4% of the accelerated pipeline.

1.3 Measured Conv2d throughput on the CPU:

FLOPs per batch (fwd+bwd, 32 images): 348.4 GFLOP (from ai_calculation.md)

Average backward time per batch:       2.442 s

Measured throughput: 348.4 GFLOP / 2.442 s = 142.7 GFLOP/s

CPU peak theoretical (Intel ARK, AVX2 boost 4.2 GHz, float32): 268.8 GFLOP/s

CPU utilization: 142.7 / 268.8 = ~53%

The bottleneck is clear: the CPU is operating at 53% of its theoretical peak on a compute-bound kernel that consumes 74.4% of training time. Custom hardware that eliminates the general-purpose CPU overhead (cache hierarchies, branch prediction, out-of-order execution) for Conv2d is the correct intervention.

1.4 Model Accuracy and Justification for Conv2d Acceleration

From `_run_outputs/run_01_output.ipynb` (representative run):

| Model | Value Accuracy | Value Loss | Train Accuracy |
| --- | --- | --- | --- |
| CNN Baseline (ResNet18, 1 epoch) | 92.37% | 0.1846 | 85.52% |
| HybridModel (ResNet18, Attentionfusion, 1epoch) | 89.92% | 0.2212 | 85.35% |

Attention weight analysis (sample inference, `0398_08_a.png`):

| Feature Source | Weight |
| --- | --- |
| CNN features (ResNet18 backbone) | 98.94% |
| Handcrafted Features (GLCM, Morphology, color) | 1.06% |

The model relies on the CNN visual path for 98.94% of its classification decision. This directly confirms that accelerating the ResNet18 Conv2d layers (the dominant component of the CNN path) is the correct hardware target — the handcrafted feature path contributes negligibly and does not merit hardware acceleration.

The M1 hypothesis was: build a systolic array delivering >= 1 TFLOP/s, achieve >= 3x system-level speedup (Amdahl, f = 74.4%). At 1 TFLOP/s the projected kernel speedup was 7× and system speedup 2.76×.

M4 Result: 2.304 TFLOP/s, 16.1× kernel speedup, 3.31× system speedup — both targets exceeded because 1024 tiles were implemented rather than the minimum 444 tiles needed to reach 1 TFLOP/s.

SECTION 2: ROOFLINE ANALYSIS

Arithmetic intensity of the Conv2d kernel was calculated analytically from ResNet18 layer dimensions (torchinfo, batch_size=1, input=224x224x3) and verified against cProfile measurements.

2.1 FLOP count (forward + backward, all Conv2d layers, 1 image):

Total Mult-Adds (torchinfo): 1,814,511,616

FLOPs (forward only):        2 x 1,814,511,616 = 3,629,023,232

FLOPs (fwd + bwd, 3x fwd):  3 x 3,629,023,232 = 10,887,069,696

2.2 Bytes transferred (float32, no DRAM reuse assumed):

Weights (all 20 Conv2d layers):   45,648,896 bytes

Input activations:                23,986,688 bytes

Output activations:               23,592,960 bytes

Forward total:                    93,228,544 bytes

Fwd + bwd (grad storage adds 2x weights + 2x activations): 232,106,048 bytes per image

2.3 Per batch (batch_size=32):

FLOPs:  348,386,230,272 (348.4 GFLOP)

Bytes:  7,427,393,536 (7.43 GB)

2.4 Arithmetic Intensity:

AI = 348.4 GFLOP / 7.43 GB = 46.9 FLOP/byte  (fwd + bwd, per batch)

AI = 3.63 GFLOP / 93.2 MB  = 38.9 FLOP/byte  (fwd only, 1 image)

2.5 CPU roofline parameters (Intel i5-10210U):

Peak compute:  268.8 GFLOP/s (AVX2, 4.2 GHz boost)

Memory BW:     45.8 GB/s (DDR4-2667, dual-channel)

Ridge point:   268.8 / 45.8 = 5.9 FLOP/byte

At AI = 46.9 FLOP/byte >> ridge 5.9 FLOP/byte, the Conv2d kernel is strongly COMPUTE-BOUND on the CPU. At AI = 46.9 FLOP/byte < ridge 50.3 FLOP/byte, the kernel approaches memory-bound on the co-processor — however, weight-stationary reuse (weights loaded once, broadcast to all 1024 PEs) reduces effective bytes/FLOP, keeping the design in the compute-bound regime in practice.

2.6 Operating Points (see Figure 2.6(a) or `bench/roofline_final.png`)

| Point | AI (FLOP/byte) | Performance |
| --- | --- | --- |
| M1 CPU(measured, cProfile) | 46.9 | 142.7 GFLOP/s |
| M4 single tile (Cadence Synthesis) | 46.9 | 2.25 GFLOP/s |
| M4 32x32 array (synthesis validated) | 46.9 | 2,304GFLOP/s |

Fig 2.6(a) Final Roofline

This is close to the DDR4 ceiling (45.8 GB/s), meaning the design approaches the memory-bound boundary. In practice, weight reuse across IFM tiles (weights loaded once and broadcast to all 1024 PEs simultaneously) reduces effective bytes/FLOP significantly, keeping the design in the compute-bound regime under realistic

inference and training workloads.

2.7 System speedup via Amdahl's Law (f = 74.4% accelerated fraction):

System speedup = 1 / ((1 - 0.744) + 0.744 / 16.1)

= 1 / (0.256 + 0.046)

= 1 / 0.302

System speedup = 3.31×

The roofline analysis shaped two key architectural decisions:

1. Weight-stationary dataflow: reuse weights across all 1024 output tiles to reduce effective bandwidth demand and push toward compute-bound operation.

2. 1024-tile array (32x32): scale compute to 2.304 TFLOP/s to stay above the 1 TFLOP/s M1 hypothesis target while keeping bandwidth within DDR4 limits.

SECTION 3: PRECISION AND DATA FORMAT

The data format evolved across three milestones:

3.1 Format Evolution

| Milestone | Format | Width | Reason for Change |
| --- | --- | --- | --- |
| M1 | Q16.16 fixed-point | 32 bits | Baseline; silent overflow risk, 2x bandwidth vs FP16 |
| M2 | FP16 (IEEE754) | 16 bits | 2x bandwidth gain, floating exponent, industry standard |
| M3/M4 | BF16 | 16 bits | No exponent rebasing hardware, TPU/A100 alignment |

3.2 BF16 Format Detail

BF16: 1 sign + 8 exponent (bias=127) + 7 mantissa

FP16: 1 sign + 5 exponent (bias=15)  + 10 mantissa

FP32: 1 sign + 8 exponent (bias=127) + 23 mantissa

Key advantage of BF16 over FP16: BF16 shares the same 8-bit exponent field as FP32 (bias=127). Converting BF16→FP32 requires only zero-extending the mantissa from 7 to 23 bits — no exponent rebasing. FP16→FP32 requires adding 112 (= 127 − 15) to every exponent, adding a hardware adder on the critical path.

BF16 SQNR: 6.02 × 7 + 1.76 ≈ 44 dB (above the 40 dB minimum for CNN classification)

FP16 SQNR: 6.02 × 10 + 1.76 ≈ 62 dB

Worst-case accumulated error (deepest layer, 4,608 MACs): 4,608 × 1.19×10⁻⁷ ≈ 5.5×10⁻⁴ — three orders of magnitude below classification output tolerance.

3.3 M4 INT32 Block-Float Accumulator

The M4 `compute_core_ti` implements a block-float INT32 accumulator in place of a floating-point adder tree:

1. `bf16_mul` function: BF16 × BF16 → FP32 (sign / exp / mantissa)

2. Find maximum exponent across all 9 products

3. Right-shift all mantissas to align to maximum exponent

4. Sum aligned mantissas in a 4-level binary adder tree (INT32)

5. Single normalize + round → FP32 output

This avoids 9 independent FP32 adders; only one normalize step is needed, significantly reducing area.

SECTION 4: DATAFLOW AND ARCHITECTURE

4.1 Dataflow Pattern: Weight-stationary

In the weight-stationary pattern, filter weights are loaded once and held fixed in each PE while input activations (IFMs) flow through. For a 3x3 Conv2d kernel:

- 9 BF16 weights (144 bits) are broadcast to all 1024 PEs simultaneously via a shared weights_packed[143:0] bus in pe_array.sv

- Each PE receives its own 3x3 IFM window (144 bits) extracted combinationally from the 34x34 BF16 IFM scratchpad (ifm_buffer.sv)

- All 1024 PEs compute their dot product independently and in parallel

This is optimal for Conv2d over a full output feature map: the 9 weights are amortized across 1024 output tiles per invocation, reducing weight bandwidth to 1/1024 of the naive case.

Alternative Patterns Considered:

- Output-stationary: partial sums remain in PE; requires accumulation across channels; not suited to single-channel 3x3 with 9 MACs per PE.

- Input-stationary: IFMs held fixed, weights rotate; wastes bandwidth for standard inference where the same weights apply to many input tiles.

Weight-stationary was chosen because the 9 BF16 weights broadcast over a 144-bit bus cost zero per-PE overhead, and the 34x34 IFM buffer enables combinational window extraction without SRAM latency.

4.2 Module Hierarchy:

The module hierarchy is shown in Figure 4.2(a):

Fig 4.2(a) Module Hierarchy

4.3 Persistent Weight SRAM (sram_sp.sv)

The `sram_sp.sv` module implements a 9×16-bit single-port SRAM that holds the 9 BF16 filter weights between kernel invocations.

Key properties:

- Weights are written once via AXI4-Stream (one 144-bit beat packs all 9 BF16 values) and held in the SRAM until the next weight-load command.

- The SRAM has an asynchronous read port — weight delivery to the compute core is single-cycle with no read latency.

- A look-ahead address scheme during the sequential write state (`sram_addr = seq_cnt + 1` during SEQ_DRIVE) allows single-cycle weight handoff without pipeline stalls.

- This persistent storage enables weight reuse across multiple IFM tiles without re-sending weights over AXI each time — the fundamental mechanism behind weight-stationary efficiency.

4.4 Backward Pass — grad_core.sv

The `grad_core.sv` module computes 18 weight and activation gradients combinatorially in a single clock cycle:

- dL/dW[i] = upstream gradient dL/dy × BF16(x[i]) for i = 0..8 (9 weight gradients)

- dL/dX[i] = upstream gradient dL/dy × BF16(w[i]) for i = 0..8 (9 input gradients)

All 18 gradients are independent single-multiplies with no data dependency, so the fully-parallel combinational implementation costs nothing in latency beyond the multiply itself. A sequential version would save ~50% area but would require 9× longer backward latency — not a worthwhile trade-off at this kernel size. The Adam optimizer runs on the host CPU (FP32), receiving the 9 FP32 weight gradients via AXI4-Lite reads from the `GRAD_W0–W8` registers.

FSM latency Evolution (m3->m4)

| Milestone | FSM | Latency | Notes |
| --- | --- | --- | --- |
| M3 (single-tile) | IDLE→LOAD(×9)→COMPUTE→DONE | 11 cycles | Sequential weight loading, one weight/cycle |
| M4 (compute_core_ti) | IDLE→LOAD→PIPE_MUL→PIPE_ACC→DONE_ST | 4 cycle | Parallel load of all 9 weights in 1 cycle |

The 7-cycle reduction (11→4) was achieved by restructuring the LOAD state to latch all 9 BF16 weight/IFM pairs simultaneously from the broadcast bus, rather than sequencing through 9 individual SRAM reads.

4.5 Memory Hierarchy

| Level | Location | Content | Size |
| --- | --- | --- | --- |
| 0 | On-chip, per-PE | 9× BF16 weight latches (registered in LOAD) | 144 bits |
| 0b | On-chip, shared | sram_sp 9×16-bit persistent weight SRAM | 144 bits |
| 1 | On-chip shared | 34×34 BF16 IFM scratchpad (ifm_buffer) | 18,496 bits |
| 2 | Off-chip | DDR-2667 host memory | - |

SECTION 5: HARDWARE INTERFACE

5.1 Interface Comparison

Two interfaces were evaluated for the host-accelerator link:

| Interface | Width | Frequency | Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| AXI4-Stream | 512-bit | 500 MHz | 32 GB/s | Selected (primary data) |
| AXI4-Lite | 32-bit | 500 MHz | 2 GB/s | Selected (control/result) |
| PCIe Gen3 x4 | -- | -- | 16 GB/s | No on-chip equivalent |
| AXI4-Stream 1GHz | 512-bit | 1 GHz | 64 GB/s | M1 spec; exceeded SAED14nm |

5.2 Bandwidth Analysis:

Required bandwidth to sustain 2.304 TFLOP/s at AI = 46.9 FLOP/byte:

Peak required BW = 2304 GFLOP/s / 46.9 FLOP/byte = 49.1 GB/s

Available BW = 512 bits / 8 × 500 MHz          = 32 GB/s

The design is not AXI4-Stream interface-bound at 32 GB/s because the IFM scratchpad (34x34) acts as a staging buffer: the host loads the IFM buffer once per Conv2d invocation(37 AXI beats ≈ 4.7 KB), then all 1024 PEs read their windows combinational from on-chip SRAM without further AXI traffic.

Transfer time per full pass at 32 GB/s = 4700 B / 32e9 B/s = 147 ns

5.3 Bandwidth Margin

| Metric | Value |
| --- | --- |
| Available (AXI4-Stream, 500 MHz, 512-bit) | 32 GB/s |
| Required at M1 target (1 TFLOP/s) | 21.3 GB/s |
| Bandwidth margin | 1.5× (reduced from M1 spec's 3× at 64 GB/s) |

The design is NOT interface-bound during the compute phase.

SECTION 6: VERIFICATION

Verification was carried out across all four milestones using cocotb v2.0.1 on Icarus Verilog 11.0. The verification strategy escalated from unit tests to full end-to-end array tests.

6.1 M2 verification (FP16 single-tile unit tests):

| Test | Description | Result |
| --- | --- | --- |
| CC1 | Weights [1.0–9.0] FP16, IFMs [1.0×9], expected 45.0 | PASS |
| CC2 | Alternating-sign weights, IFMs [2.0×9], expected 2.0 | PASS |
| IF1 | AXI4-Lite write CTRL, read STATUS=done, RESULT=45.0 | PASS |
| IF2 | Weight Reload without device reset | PASS |

The M3 single-tile testbench (m3/synth/) verified the 4-cycle FSM on a single PE before scaling to 1024 tiles.

6.2 M4 verification (BF16 full 32x32 array) (see Figure 6.2(a)):

DUT: `top_ti` (N_TILES=**1024**) | Runner: `sim/run_ti.py`

| Test | Cocotb name | Description | Coverage |
| --- | --- | --- | --- |
| 1 | test_forward.test_bf16_forward | 32×32 BF16 forward pass — T1: ramp weights × unit IFM (all 1024 tiles = 45.0); T2: alternating-sign × const-2.0; T3: max BF16-safe × unit (1143.0); T4: Laplacian × ramp (per-tile varies). Checks all **1024** FP32 results vs `ref_model.py` (tol=**1e-3**) | Forward path, all tiles, register map 0x1000–0x1FFC |
| 2 | test_forward.test_weight_reuse | Loads weights `[1..9]` once, verifies 4 different 34×34 IFM tiles: unit (45.0), double (90.0), negative unit (−45.0), half (22.5) | Weight-stationary reuse, no reload between tiles |
| 3 | test_backward.test_grad_w | B1: unit IFMs, dldy=1.0 → grad_w=ifms; B2: ramp IFMs, dldy=2.0 → grad_w=2×ifms; B3: alternating IFMs, dldy=0.5; B4: Laplacian weights, ramp IFMs, dldy=−1.0 | Backward pass dL/dW for all 4 vectors |

The forward-pass waveform is shown in Figure 6.2(a).

Fig 6.2(a) Forward-pass waveform

Final result (`sim/final_run.log`):

TESTS=3 PASS=3 FAIL=0 SKIP=0 | Total simulation time: 342,140 ns

| ID | Category | Description |
| --- | --- | --- |
| B1 | Stale ports | `tb_top.sv` instantiated `conv_interface` instead of `conv_interface_ti` |
| B2 | Stale ports | `tb_interface.sv` used **4-bit** AXI4-Lite address (should be **13-bit**) |
| B3 | Stale ports | `core_result[31:0]` (should be `core_result[32767:0]` = 1024×32) |
| B4 | cocotb timing | NBA write inside `@(posedge clk)` read back one cycle late; fixed with `await RisingEdge` |
| B5 | cocotb API | Direct `signal = value` silently failed; must use `signal.value = value` |
| B6 | Double-conversion | `bf16_to_float()` called on already-float value; result off by ~32,000× |
| B7 | Icarus compat | `$bitstoreal()` unsupported inside generate blocks in Icarus 11.0 |
| B8 | Icarus compat | Parameter override syntax in generate loops caused elaboration errors |
| B9 | Icarus compat | Multi-dimensional packed array indexing requires intermediate `wire` |
| B10 | Makefile | cocotb/compute_core/Makefile` referenced `compute_core.sv` (should be `compute_core_ti`) |
| B11 | Mantissa bug | `{prod[14:1], 8'h00}` dropped LSB → ~0.4% error/MAC → failed 1e-3 tol; fixed to `{prod[14:0], 8'h00}` |
| B12 | Cocotb timing | Combinational output required `await Timer(0)` delta NOP before read |

Bug B11 was the most significant: the off-by-one in `bf16_mul` accumulated across 9 MACs, violating the 1e-3 tolerance threshold in `test_forward`. Fixed by correcting the slice from `[14:1]` to `[14:0]` in `compute_core_ti.sv`.

SECTION 7: SYNTHESIS RESULTS

The 32x32 systolic array (top_ti, N_TILES=1024) was synthesized using Cadence Genus 17.14-s037_1 (legacy_ui) on SAED14nm RVT standard cells. All numbers below are from the actual 32x32 synthesis run (not single-tile projections).

7.1 Tool Configuration:

| Parameter | Value |
| --- | --- |
| Tool | Cadence Genus 17.14-s037_1 (legacy_ui) |
| PDK | SAED14nm RVT |
| Top module | top_ti |
| Clock period | 2.0 ns (500 MHz) |
| HDL loop limit | 4096 (required for N_TILES=1024 generate loop) |
| Synthesis effort | high |

7.2 Timing(timing_report.txt)

| Metric | Value |
| --- | --- |
| Clock Period | 2000ps (500MHz) |
| Worst Negative Slack | 0 ps — MEETS CONSTRAINT |
| Total Negative Slack | 0 ps |
| Setup violation | 0 |
| Critical path | gen_tile[163] mul_prod_reg_reg → result_reg_reg |
| Critical path | 1950 ps (WNS = 0 ps; 50 ps pre-uncertainty margin) |
| Critical path route | BF16 latch → exponent adder → mantissa multiplier → normalize → result register |

7.3 Area (area_report.txt):

| Module | Cells | Area (um2) | % of total |
| --- | --- | --- | --- |
| top_ti (total) | 5,591,114 | 2,862,977 | 100% |
| u_pe_array | 5,453,789 | 2,773,992 | 96.9% |
| u_interface | 137,325 | 88,985 | 3.1% |

Total area: 2,862,977 um2 = 2.86 mm2

Per-tile average: 2,773,992 / 1024 = 2,709 um2 per PE

Dominant area contributor: pe_array (96.9%)

Within pe_array: compute_core_ti 1024x (each ~2.7 kum2)

Dominant area contributor: `pe_array` (96.9%), driven by the BF16 multiplier (7×7 unsigned mantissa product tree + normalize logic) replicated 1024×

7.4 Power (power_report.txt, TT corner, 500 MHz):

| Module | Power (mW) | % of total |
| --- | --- | --- |
| top_ti | 1,359.89 mW | 100% (1.36 W) |
| u_pe_array | 1,259.17 mW | 92.6% |
| u_interface | 43.97 mW | 3.2% |
| Other (ifm_buf etc) | 56.75 mW | 4.2% |

Per-tile average: 1,259.17 / 1024 = 1.23 mW per PE

Single-tile synthesis (from m3_ti/cadence_syn/): 3.07 mW

Difference: 1.23 vs 3.07 mW per tile -- lower in array due to Genus optimization of shared weight broadcast logic across the generate loop

COMPARISON: SINGLE TILE vs 32x32 ARRAY

| Metric | M4 Single Tile | M4 32x32 Array | Delta |
| --- | --- | --- | --- |
| WNS | +2.4 ps | 0 ps | Tighter, still meets |
| Total cells | 11,340 | 5,591,114 | x493 |
| Area | 6,155 um2 | 2,862,977 um2 | x465 (2.86 mm2) |
| Power | 3.07 mW | 1,359.89 mW | x443 (1.36 W total) |
| Per-tile power | 3.07 mW | 1.23 mW | 0.4x (40% of single) |
| Throughput | 2.25 GFLOP/s | 2,304 GFLOP/s | x1024 |

SECTION 8: BENCHMARK RESULTS

All benchmark numbers are synthesis-validated from the 32x32 run reports.

Raw data is in ‘bench/benchmark_data.csv’.

8.1 Throughput:

Per-tile: 9 MACs x 2 FLOPs x 500 MHz / 4 cycles = 2.25 GFLOP/s

32x32 array: 1024 x 2.25 = 2,304 GFLOP/s = 2.304 TFLOP/s

8.2 Kernel Speedup (Conv2d):

| Platform | Throughput | Speedup vs M1 |
| --- | --- | --- |
| M1 CPU (measured, cProfile) | 142.7 GFLOP/s | 1× (baseline) |
| M4 single tile (synthesis) | 2.25 GFLOP/s | 0.016× |
| M4 32x32 array (synthesis-validated) | 2,304 GFLOP/s | 16.1× |

8.3 System Speedup (Amdahl's Law):

Accelerated fraction:  f = 74.4%  (Conv2d fwd+bwd, cProfile)

Kernel speedup: 16.1x

System speedup = 1 / ((1 - 0.744) + 0.744 / 16.1)

= 1 / (0.256 + 0.046)

= 1 / 0.302

= 3.31x

Wall-clock per epoch (baseline):   2,573.9 s (42.9 minutes)

Wall-clock per epoch (with array): 2,573.9 / 3.31 = ~777 s (~13.0 minutes)

Pipeline throughput (baseline):    3.73 samples/sec

Pipeline throughput (with array):  3.73 x 3.31 = ~12.3 samples/sec

8.4 Energy Efficiency:

| Platform | Power | Throughput | Energy/FLOP |
| --- | --- | --- | --- |
| M1 CPU | ~15 W TDP | 142.7 GFLOP/s | 105 pJ/FLOP |
| M4 single tile | 1.23 mW | 2.25 GFLOP/s | 0.55 pJ/FLOP |
| M4 32x32 array | 1.36 W | 2,304 GFLOP/s | 0.59 pJ/FLOP |
| NVIDIA T4 GPU | 70 W | 65,000 GFLOP/s | ~1.08 pJ/FLOP |

Energy efficiency vs CPU: 105 / 0.59 = 178x more efficient per FLOP

Energy efficiency vs T4:  1.08 / 0.59 = 1.8x more efficient per FLOP

Power vs T4: 70 / 1.36 = 51x less power

The 178x energy advantage over the CPU arises from two sources:

1. SAED14nm FinFET process: per-gate switching energy is ~10-20x lower than the 14nm Intel FinFET in the i5-10210U at the same logic function.

2. Dedicated BF16 datapath: no cache hierarchy, branch predictor, or out-of-order execution logic is toggling on every MAC operation.

8.5 Performance Gaps:

1. Pipeline utilization(50%): each PE spends 2 of 4 cycles computing (PIPE_MUL + PIPE_ACC) and 2 cycles on overhead (LOAD + DONE_ST). Utilization = 50%. A deeper pipeline or output-stationary accumulation across channels would improve this toward 100%.

2. AXI4-Stream bandwidth: M1 spec assumed 1 GHz (64 GB/s). The synthesized design runs at 500 MHz (32 GB/s). The 32 GB/s exceeds the 21.3 GB/s requirement at the M1 target (1 TFLOP/s), but the 49.1 GB/s requirement for the full 2.304 TFLOP/s is not met in the worst case without reuse. Weight reuse from the broadcast bus eliminates this in practice.

8.6 M1 Hypothesis Evaluation:

The M1 roofline (`partition_rationale.md`, `interface_selection.md`) projected a 7× kernel speedup and 2.76× system speedup at the 1 TFLOP/s design target:

M1 projection:  Speedup = 1 / (0.256 + 0.744 / 7)  = 2.76×

M4 actual:      Speedup = 1 / (0.256 + 0.744 / 16.1) = 3.31×

| Metric | M1 Projected (1TFLOP/s) | M4 Actual (2.304 TFLOP/s) | Status |
| --- | --- | --- | --- |
| Compute Target | ≥ 1 TFLOP/s | 2.304 TFLOP/s | Exceeded ×2.3 |
| Kernel Speedup | 7× (roofline) | 16.1× | Exceeded ×2.3 |
| System speedup | 2.76× (Amdahl) | 3.31× | Exceeded |
| Interface BW | 64 GB/s @ 1 GHz | 32 GB/s @ 500 MHz | Reduced (SAED14nm limit) |

The over-achievement in compute is due to implementing 1024 tiles (32×32) rather than the minimum 444 tiles (= 1 TFLOP/s ÷ 2.25 GFLOP/s per tile) needed to meet the M1 target. The interface bandwidth reduction (64→32 GB/s) is acceptable because the 34×34 IFM scratchpad absorbs the burst load; sustained effective bandwidth during compute is near zero (weights already loaded, IFM already on-chip).

Three deviations from M1 plan (with justification):

| Deviation | M1 Plan | M4 Actual | Reason |
| --- | --- | --- | --- |
| Data Format | Q16.16 -> FP16 | BF16 | Eliminates exponent rebasing adder on critical path |
| Synthesis Tool | Openlane/ sky130A | Cadence Genus (node-SAED 14nm) | OpenLane could not close 1024-tile design (130 ns best) |
| Synthesis Scope | Single-tile proof | Full 32x32 array | Direct synthesis of production array at SAED14nm |

SECTION 9: WHAT DID NOT WORK

9.1 Openlane 2 / Sky130A Failed To Close Timing (complete failure)

Attempted: OpenLane 2.3.10 with sky130A (sky130_fd_sc_hd) PDK for both M3 single-tile and M4 32x32 array synthesis.

Single-tile result (m3/synth/openlane_run.log):

TT corner WNS: +29.9 ns (PASS)

SS corner WNS: -63.0 ns (FAIL) - expected due to FP32 datapath depth; sky130A SS corner is highly pessimistic for floating-point chains.

32x32 array: OpenLane could not close the 32x32 array at any clock period faster than 130 ns (7.7 MHz). Root causes:

a. sky130_fd_sc_hd has ~2-3x higher cell delays than SAED14nm RVT (180 nm bulk vs 14 nm

FinFET process)

b. The N_TILES=1024 generate loop created routing congestion the OpenLane placer could not

resolve within the **800×800 µm** die

c. 32×32 array: OpenLane could not close the design at any clock period faster than **130 ns

(7.7 MHz).

Resolution: switched to Cadence Genus 17.14 on SAED14nm RVT PDK via the PSU auto.ece.pdx.edu server, which closed timing at 500 MHz (WNS = 0 ps).

9.2 BF16_MUL Mantissa off-by-one (Bug B11)

The bf16_mul function in compute_core_ti.sv originally used:

{prod[14:1], 8'h00}    -- WRONG (drops LSB of 15-bit product)

This should have been:

{prod[14:0], 8'h00}    -- CORRECT (all 15 bits, then pad to FP32 mantissa)

The off-by-one caused ~0.4% relative error per MAC, which accumulated over 9 MACs to produce results that failed the 1e-3 tolerance in test_forward.py. Detected by: comparing against ref_model.py golden output with the forward test vectors. Fixed by correcting the slice from [14:1] to [14:0].

9.3 COCOTB NBA TIMING (Bugs B4, B5, B12)

cocotb v2.0.1 changed signal assignment semantics vs v1.x:

- Assignments inside @(posedge clk) coroutines use NBA (non-blocking

assignment) scheduling; reading back the assigned value requires

awaiting a delta cycle before the new value is visible to Python.

- Direct signal assignment (signal = value) silently failed; must use

signal.value = value.

- Combinational outputs required an explicit await Timer(0) (delta NOP)

after driving inputs before reading outputs.

Resolution: added await RisingEdge(dut.clk) after each control write, and await Timer(1, units='ns') before reading combinational outputs.

9.4 Stale ports in legacy testbenches (Bugs B1, B2, B3)

tb/tb_interface.sv was written for the M2 single-tile conv_interface module:

- AXI4-Lite address: 4-bit (should be 13-bit for 32x32 register map)

- core_result width: 32-bit (should be 32768-bit = 1024 x 32)

- Module name: conv_interface (should be conv_interface_ti)

Resolution: tb/tb_interface.sv was excluded from the M4 testbench suite; tb_top.sv

(which correctly instantiates top_ti) is the M4 primary testbench.

9.5 DOUBLE-CONVERSION BUG (Bug B6)

In `test_backward.py`, `bf16_to_float()` was called on a value that was already a Python float. The function interpreted the float's integer representation as a BF16 bit pattern, producing a golden value off by a factor of ~32,000. Fixed by tracking data types through the call chain.

9.6 ICARUS VERILOG COMPATIBILITY (Bugs B7, B8, B9)

| Bug | Issue | Fix |
| --- | --- | --- |
| B7 | `$bitstoreal()` unsupported inside generate blocks in Icarus 11.0 | Replaced with explicit sign/exp/mantissa unpacking (synthesizable) |
| B8 | Parameter override syntax in generate loops caused elaboration errors | Replaced with explicit `localparam` per generate iteration |
| B9 | Multi-dimensional packed array indexing (`packed_bus[t*144 +: 144]`) requires intermediate `wire` | Added `wire [143:0] w_ifm_t` per generate iteration |

9.7 Window encoding bug in unicode test names (Bug B10)

test_backward.py used Unicode right-arrow character (U+2192: ->>) in test vector names ("B1: unit IFMs, dldy=1.0 -> grad_w = ifms"). On Windows with cp1252 encoding, cocotb's log output raised:

UnicodeEncodeError: 'charmap' codec can't encode character '→'

Fixed by replacing -> with ASCII -> in all test name strings.

WHAT WOULD BE DONE DIFFERENTLY:

- Start with Cadence Genus on SAED14nm from the beginning rather than attempting OpenLane/sky130A for a 1024-tile design; sky130A is appropriate for single-tile proof-of-concept but cannot handle the cell count.

- Formally verify `bf16_mul` mantissa alignment from the start, rather than catching Bug B11 late in cocotb testing.

- Use a cocotb compatibility shim layer for signal assignment to isolate version-specific NBA timing behavior from test logic.

RAW DATA REFERENCES

All numbers in this report trace to the following committed files:

| File | Content |
| --- | --- |
| bench/benchmark_data.csv | All raw numbers: M1_baseline, M4_single_tile, M4_array_32x32, benchmark_comparison |
| bench/roofline_final.png | Roofline plot: CPU/co-proc curves + 3 operating points |
| synth/timing_report.txt | WNS=0 ps, 10 timing paths, 500 MHz |
| synth/area._report.txt | 5,591,114 cells, 2,862,977 µm² |
| synth/power_report.txt | 1,359.89 mW total (TT corner) |
| sim/final_run.log | TESTS=3 PASS=3 FAIL=0, 342,140 ns |
| project/m1/sw_baseline.md | 10-run wall-clock table, cProfile data |
| project/m2/precision.md | FP16 SQNR, error analysis, verification |
| project/m3_ti/synthesis_notes_ti.md | Single-tile design decisions |

Figures

| Figure | Location |
| --- | --- |
| Fig 2.6(a) Final Roofline | m4/bench/roofline_final.png |
| Fig 4.2(a) Module Hierarchy | m4/report/design_justification.pdf |
| Fig 6.2(a) Forward-pass waveform | m4/sim/final_waveform.png |

Extra Figures (present in m4/report/figures)

| Figure | Location |
| --- | --- |
| ExtraFig1 sample_anemic_image | m4/report/figures/sample_anemic_image.png |
| ExtraFig2 sample_healthy_image | m4/report/figures/sample_healthy_image.png |
| ExtraFig3 post_training_inference | m4/report/figures/post_training_inference.png |
| ExtraFig4 hotspot_analysis | m4/report/figures/hotspot_analysis.png |
| ExtraFig5 confusion_matrix_anemia_core_kernel | m4/report/figures/confusion_matrix_anemia_core_kernel.png |